HACETTEPE UNIVERSITY

DEPARTMENT OF ELECTRICAL AND ELECTRONICS ENGINEERING

ELE 432: Advanced Digital Design

Experiment 3 / Preliminary Work

Hakan Töre

2210357024

- This lab took me around 6 hours. Since I have already finished the controller circuit for the previous homework.

| Step | PC | Instr | State | Result | Result Notes |
| --- | --- | --- | --- | --- | --- |
| 3 | 00 | 00500113 | S0: Fetch | 4 | PC+4 |
| 4 | 04 | "" | S1: Decode | X | OldPC+Immediate |
| 5 | 04 | "" | S8: ExecuteI | X | ALUResult = x0 (0) + 5 = 5 |
| 6 | 04 | "" | S7: ALUWB | 5 | Result = ALUOUT |
| 7 | 04 | "" | S0: Fetch | 8 | PC+4 |
| 8 | 08 | 00c00193 | S1: Decode | X | OldPC+Immediate |
| 9 | 08 | "" | S8: ExecuteI | X | ALUResult = x0 (0) + 12 = 12 (0xc) |
| 10 | 08 | "" | S7: ALUWB | 12 | Result = ALUOUT |
| 11 | 08 | "" | S0: Fetch | Ch | PC+4 |
| 12 | 0c | ff718393 | S1: Decode | X | OldPC+Immediate |
| 13 | 0c | "" | S8: ExecuteI | X | ALUResult = x3 (12) + (-9) = 3 |
| 14 | 0c | "" | S7: ALUWB | 3 | Result = ALUOUT |
| 15 | 0c | "" | S0: Fetch | 10h | PC+4 (Starts next instruction) |

https://github.com/Hakan-26/Multicycle-Processor-Full